   Cessna 172F
              Preflight
                8716U                   In Flight
    Procedures             Emergency              Emergency             Reference


Preflight Cockpit

Aircraft docs (ARROW)
Check
Weight & Balance
Check
Parking Brake
Set
Control wheel lock
Remove
Hobbs/Tach
Check/Record
Ignition
Off
Avionics Power Switch
Off
Master Switch
On
Fuel quantity indicators
Check
Flaps 
Extend

Lights
On/Check/Off
Master Switch
Off

Fuel Strainer
Pull
Fuselage and Empennage
Baggage Door
Closed & Locked
Tail Tie-Down
Disconnect
Control surfaces
Free & Secure
Trim Tab
Check Security

Antennas
Check Security
Right wing

Wing tie-down
Disconnect
Aileron
Free and Secure
Flaps
Secure
Main wheel tire(23psi)
Inflated/Cond
Brakes
Pads/Leaks
Fuel tank sump
Sample 

Fuel quantity
Check
Fuel filler cap
Secure
Nose

Air inlet covers
Remove
Propeller and spinner
Check
Air intake 
Check
Carburetor air filter
Check
Nose wheel strut & tire(26psi)
Check
Static source opening
Check

Engine oil level (6 Quarts)
Check
            (Phillips 66 X/C 20W50)
Left Wing

Wing tie-down
Disconnect

Aileron
Free and Secure
Flaps
Secure
Main wheel tire(23psi)
Inflated/Cond
Brakes
Pads/Leaks
Fuel tank vent open
Check
Fuel tank sump
Sample 

Fuel Quantity
Check
Fuel Filler cap
Secure
Pitot tube
 Check

Pitot flap
Free to move
Stall warning
Check
Before starting engine

Preflight inspection
Complete

Passenger Briefing
Complete
Seats, belts
Adjust & Lock

Doors 
Closed & Locked

Brakes 
Test & Set

Circuit breakers
Check In

Electrical Equip
Off

Avionics Power Switch
Off

Fuel Selector Valve
Both
Starting Engine

ENGINE COLD

Throttle
Open 1/8 inch
Mixture 
Rich (IN)
Carb heat
Cold (IN)
Prime
As required; locked

Propeller area
Clear

Master Switch
On

Beacon 
On

Ignition
Start
ENGINE WARM

Throttle
Pump Twice

Open 1/8 inch

Mixture
Lean (OUT)
Propeller area
Clear
Master Switch
On
Beacon
On
Ignition
Start

Mixture
Full Rich
Throttle
Adjust 1000 rpm
Oil Pressure
Check normal
Alternator Switch
On
Avionics Master Switch
On

Radios
On
Transponder
Standby
Flaps
Up

Ammeter 
Check

Heading Indicator 
Set

ATIS/AWOS/ASOS 
Obtain

Altimeter 
Set 
Before Takeoff

Parking brake
Set

Cabin doors
Closed & Locked

Seats, belts
Adjust & Lock
Flight controls
Free & Correct
Instruments
Set

Fuel Quantity
Check
Mixture
Rich (IN)
Fuel Selector Valve 
Both
Elevator Trim
Set for TAKEOFF
Throttle
1600 rpm

Magnetos (75 rpm max dif)
Check

Suction gage 
Check

Engine Instruments
Check

Generator
Check


Mixture
Set for Density Alt


Carb heat
On
Throttle
Idle
Carb Heat
Off (In)
Throttle Friction Lock
Adjust

Strobe Lights
On
Radios/Avionics
Set

Transponder
Test/Altitude
Flaps
Set for Takeoff (00-100)
Parking Brake
Release

Windows
Closed
 Standard Takeoff

Flaps
Up
Carb heat
Off (In)
Transponder
Altitude
Trim
Set for TAKEOFF

Throttle
Full
Tach, oil, airspeed
Check
Elevator
Lift at 60 MPH
Climb
80 MPH
Cruise

Pitch
Set
Lights
As Required
Throttle
As required
Trim
Set
Mixture 
Adjust

Fuel Selector Valve
Both

       >5000 ft elev
Left/Right
Pre-landing checklist

Fuel selector
On Both
Mixture
Rich
Carb Heat
On
Seatbelts
Fastened
Approach

Flight instruments
Ckd & Set
Radios
Checked
ATIS
Checked
Carb Heat
On (Out)
Mixture
Rich

Landing light 
On

Airspeed 
70-80 MPH (Flaps Up)

65-75 MPH (Flaps Dn)

After landing

Flaps
Up
Carb Heat
Cold (In)
Transponder
Standby
Parking

Lights
Off
Avionics
Off
Electrical
Off
Throttle
Idle
Mixture 
Cut-off
Ignition switch
Off
Master switch
Off
Securing the aircraft

Control Lock
Install
Air Inlet Cover
Install
Hobbs/Tach
Record
Door/Window
Secure

Tie-downs
Secure

Doors/Baggage Door
Locked

BUGS CLEANED OFF WINDSHIELD AND WINGS

Aircraft Key
In Locker
Locker 
Locked
Short field take-off

Take-off checklist
Complete
Flaps
10(
Taxi
Max runway
Brakes
Set and hold
Throttle 
Full
Brakes
Release
Climb
65 MPH
Flaps
Retract
Short field landing

Pre-landing check
Complete
Approach
67 MPH
Flaps
40(
Throttle
Maintain glide
Touchdown
Power Off
Flaps
Up
Elevator 
Full up
Braking
Heavy as required
Soft field take-off

Take-off checklist
Complete
Flaps
10(
Taxi
Keep rolling

Liftoff
ASAP
Level
Stay in ground effect 
Climb
80 MPH
Flaps
retract
Soft field landing

Pre-landing check
Complete
Throttle
1500 RPM
Flaps
40(
Airspeed
69 MPH
Touchdown
Main first, softly
Landing roll
Nose wheel up
Elevator
Up
Braking
As required
Go-around

Throttle
Full
Carb Heat
Cold (In)

Flaps
20(
Climb 
65 MPH
Flaps
10(
Climb
80 MPH
Flaps
Up
Engine failure

TAKEOFF

Throttle
Idle
Brakes
Apply
Flaps
Retract
Mixture
IDLE cut-off
Ignition
Off
Master switch
Off
AFTER TAKEOFF

Airspeed
80 MPH (flaps UP)
Mixture 
Idle Cut-off
Fuel selector valve
Off 
Ignition
Off
Flaps 
As Required
Master switch
Off
Cabin Doors
Unlatch
Land
Straight Ahead
DURING FLIGHT 

Airspeed 
80 MPH
LOOK FOR A FIELD
Fuel selector valve
Both
Primer 
In & Locked

Mixture
Rich
Ignition
BOTH (or START)
Airspeed
70-80 MPH (flaps UP)

65-75 MPH (flaps DN)
Mixture 
Idle Cut-off
Fuel selector valve
Off 
Ignition
Off
Flaps 
As Required
Mayday
Transmit 121.5

Mayday
Squawk 7700
Master switch
Off
Cabin Doors
Unlatch 

Touchdown
Tail Low
Carburetor Icing

Throttle
Full
Carb Heat
On (Out)
Mixture
Adjust
Engine Roughness

Magnetos
Check
Mixture
Lean as necessary
Engine Fire during start

Crank
Continue
Power
1600 RPM (2 min)
Engine
Shut down and inspect
Engine Fire during flight

Mixture 
Idle Cut-off
Fuel selector valve
Off 
Master Switch
Off

Alternator Switch
Off
Cabin Heat/Air
Off

Airspeed
120 MPH
Electrical Fire during flight
Master Switch
Off
Ignition Switch
On
All other switches
Off
Circuit Breakers
Ident Bad Circuit
Master Switch
On
Electrical Switches
On 

(One at a time)
Spin Recovery

Ailerons
NEUTRAL
Throttle
IDLE
Rudder
Full opposite
Control wheel
Full forward
Rudder control/wheel
Neutral
Pitch
Level


Light Signals:


Signal�
On Ground�
In Flight�
�
Steady Green�
Takeoff�
Land�
�
Flashing Green�
Taxi�
Return to land�
�
Steady Red�
Stop�
Give way�
�
Flashing Red�
Clear runway�
Do not land�
�
Flashing White�
Return to ramp�
--�
�
Red/Green alternating�
WARNING – USE CAUTION�
�












Alpha


Bravo 


Charlie 


Delta 


Echo 


Foxtrot 


Golf 


Hotel 


India


Juliet 


Kilo 


Lima 


Mike 


November 


Oscar 


Papa 


Quebec 


Romeo


Sierra 


Tango 


Uniform 


Victor 


Whiskey 


Xray 


Yankee 


Zulu








V-Speeds (MPH)


Rotate�
Vr�
60�
�
Normal Climb Out�
Vclmb�
80-90�
�
Max angle


(Sea Level)�
Vx�
65�
�
Max angle


(10,000 ft)�
Vx�
71�
�
Climb rate


(Sea Level)�
Vy�
80�
�
Climb rate


(10,000 ft)�
Vy�
77�
�
Maneuver �
Va�
59-122�
�
Flaps �
Vfe�
100�
�
Normal max �
Vno�
140�
�
Never exceed�
Vne�
174�
�
Stall (clean)�
Vs�
57�
�
Stall (land)�
Vso�
49�
�
Final Approach �
Flaps�
65-75�
�
�
No flaps�
70-80�
�
Max Glide�
80�
�



Useable Fuel Capacity 38 Gallons








